Antenna Rotator Calibration Procedure

Pre-setup: The following procedures will require at least one person inside the lab with access to the rotator controller and at least one person on the roof of the PSB. These two groups must have active communication with each other.

Tools needed: (unknown for adjusting mounting pole)

- Set rotator to be facing downwards (altitude/elevation at 0 degrees) using the rotator controller (down arrow).

- Confirm that the antennas on the mount bar are parallel with the horizon (0 degrees altitude/elevation).

  - If the antennas are not properly horizontal, the mounting bar must be loosened and rotated until they are. Afterwards retighten it.

- Turn the rotator to be facing true north (azimuth zero degrees) using the rotator controller (left or right arrows).

- Resetting the controller:

  - Turn the controller unit off

  - Hold the “F” button depressed, turn controller back on

  - Display should show: 0.0, 0.0

- At this point, the azimuth has been successfully calibrated

- Using the controller down arrow, move the rotator to the full travel which should be around –21.0. If the rotator stops and the display is not –21.0 then the mechanical stop in the rotator has been triggered

- Reset the controller (refer to step 4)

- Using the controller down arrow, move the rotator to the full travel downward (which should be about –21.0).

- Repeat steps 6-8 until there is no more travel.

- Using the controller up arrow, move the rotator up until it reaches the 10-degree mark.

- Reset the controller (refer to step 4)

- Now test for a full 180 degrees of travel by using the controller up arrow.

  - If travel is ~180 degrees or more then setup is correct.

  - If not, repeat the process until it is. (steps 6-12)

- The rotator is now fully calibrated.

Notes: While on the roof, we should WD40 maintenance spray the motors.